9. System Requirements & The User Workflow

9.1 The User Workflow

The system's requirements are dictated by the distinct operational workflows of our on-site and remote stakeholders:

- Phase 1: On-Site Capture & Real-Time Validation (The Supervisor): The Construction Supervisor opens the Android app in an unfinished room. Before and during recording, the app analyzes the camera feed in real-time. If the room is too dark or the supervisor walks too fast, the app immediately flashes a warning ("Low Light: Enable Flashlight" or "Motion Blur: Slow Down"), preventing a useless scan. Once a clean 60-second video is captured, it uploads silently in the background.

- Phase 2: Cloud Processing (Automated): The backend receives the video and automatically spins up an Ubuntu-based Dockerized GPU worker. It fuses the ARCore data, trains the 3D Gaussian point cloud, runs a 2D CNN to detect materials like drywall and wiring, and compresses the output.

- Phase 3: Remote Verification (The Project Manager): The Project Manager opens the Next.js web dashboard. They navigate the 3D room, click an "X-Ray / Rough-In" toggle to reveal wiring hidden behind finished walls, and click the wiring to extract its exact physical dimensions.

9.2 Functional Requirements (FRs)

Workflow A: Edge Capture & Validation

- FR1 (ARCore Telemetry Extraction): The Android application shall record 1080p video while simultaneously extracting 6-DoF ARCore spatial anchors and IMU telemetry into a synchronized JSON file.

- FR2 (Real-Time Capture Validation): The mobile client shall implement edge-based Image Quality Assessment (IQA) using Laplacian variance (for motion blur) and lux estimation (for exposure), providing real-time UI warnings to correct user movement or lighting prior to upload.

- FR3 (Asynchronous Upload): The mobile application shall upload the raw video and telemetry data directly to object storage in the background without blocking the user interface.

Workflow B: Distributed Orchestration & Reconstruction

- FR4 (Distributed Job Queuing): The Node.js backend shall enqueue uploaded scan data into a Redis message broker to asynchronously provision and orchestrate Dockerized GPU cloud workers.

- FR5 (Absolute Spatial Anchoring): The cloud pipeline shall fuse the ARCore tracking data with the video frames (via COLMAP) to lock the generated 3D Gaussian point cloud to an absolute physical coordinate grid.

- FR6 (3DGS Reconstruction): The GPU worker shall execute the 3D Gaussian Splatting training loop to transform the 2D video and ARCore poses into a raw, photorealistic 3D point cloud.

- FR7 (2D-to-3D Semantic Projection): A 2D CNN shall detect specific construction materials in video keyframes and mathematically project those bounding boxes onto the 3D Gaussians to label the geometry.

- FR8 (Heuristic Pruning & Compression): The post-processing script shall algorithmically delete low-opacity artifact splats and compress the .ply file into a web-optimized .ksplat binary.

Workflow C: Remote Delivery & Measurement

- FR9 (Progressive Web Rendering): The Next.js dashboard shall stream the .ksplat files into a custom Three.js WebGL viewer for continuous, 360-degree first-person navigation.

- FR10 (Semantic Dimension Extraction): The web viewer shall calculate and display physical surface area and length measurements when a user selects a semantically labeled material.

- FR11 (Temporal "X-Ray" Overlay): The web viewer shall allow users to overlay semantic bounding boxes from historical scans (e.g., exposed pipes) directly onto current scans (e.g., finished walls) utilizing the shared ARCore coordinate grid.

9.3 Measurable Non-Functional Requirements (NFRs)

- NFR1 (Capture Efficiency): The mobile application must maintain a stable video capture rate of ≥30 FPS while processing background telemetry.

- NFR2 (Edge Processing Latency): The real-time motion blur and lighting detection algorithms must execute on the Android client in <16 milliseconds per frame to prevent dropped frames.

- NFR3 (Pipeline Latency): End-to-end cloud processing for a 60-second video must finish in ≤15 minutes on an RTX 4090 cloud instance.

- NFR4 (Compression Ratio): The .ksplat binary compression must reduce the raw 3D file payload by ≥60% for web delivery.

- NFR5 (Client Accessibility): The Next.js web viewer must utilize an adaptive splat budget to maintain ≥30 FPS on standard mid-range browsers.

- NFR6 (Dimensional Accuracy): Physical measurements extracted from the 3D geometry must achieve an error variance of ≤5% compared to physical laser tape measurements.

10. Scope, Risks, and Dependencies

10.1 Committed Core Scope & Semester Timeline

To ensure a verifiable grading standard, the scope is strictly divided across two semesters.

Semester 1 (FYP-1): Core Build & Cloud Pipeline

- Mobile Capture: Android application with ARCore telemetry extraction and the edge-based Image Quality Assessment (IQA) module for lighting/blur detection.

- Cloud Orchestration: Node.js/Express backend utilizing Redis to handle asynchronous job queuing.

- Infrastructure: Setting up Dockerized cloud environments on Ubuntu Linux to handle heavy parallel compute tasks.

- Reconstruction Engine: Fusing ARCore telemetry with COLMAP and generating a raw 3D Gaussian point cloud.

- Exit Gate (Final-1): A working core pipeline that successfully takes a mobile video, validates its quality, and generates a mathematically scaled 3D scene.

Semester 2 (FYP-2): Intelligence, Optimization, & Delivery

- Compression Engineering: Developing the heuristic pruning scripts to convert massive .ply files into .ksplat binaries.

- Semantic AI Integration: Implementing the 2D-to-3D projection scripts that map CNN material labels onto the 3D geometry.

- Web Dashboard: Building the Next.js and MongoDB web portal featuring the adaptive Three.js viewer.

- Evaluation: Executing formal User Acceptance Testing (UAT) with the named construction stakeholder.

- Exit Gate (Final-2): A fully integrated, deployed web platform with verifiable dimension extraction, passing UAT.

10.2 Dependency Classification

- Support: Next.js, Node.js, Express, MongoDB, Ubuntu Linux, Docker, Redis, Cloudflare R2 SDK. (These provide standard scaffolding and do not solve the core Complex Computing Problem).

- Core-Assist: COLMAP (for base SfM), 3DGS CUDA kernels, standard YOLO/Mask2Former CNN weights, Three.js. (We are responsible for integrating, modifying, and orchestrating these tools).

- Core-Replacement: None. (The ARCore multi-sensor fusion, edge IQA algorithms, asynchronous orchestration, .ksplat compression, and geometric 2D-to-3D semantic mapping are engineered entirely by the team).

10.3 Initial RAID Register

| Type | Description | Mitigation Strategy |
| --- | --- | --- |
| Risk | User captures unusable, blurred, or dark video, wasting expensive GPU cloud compute time. | Implemented edge-based Image Quality Assessment (FR2) to block bad uploads and enforce real-time capture correction. |
| Risk | ARCore hardware telemetry drifts over time, causing spatial warping in the 3D model. | Limit mobile capture sequences to 60-second chunks to prevent accumulated IMU drift and force periodic spatial resets. |
| Assumption | Cloud GPU instances (RunPod) will remain highly available. | The pipeline is heavily containerized (Docker); execution can seamlessly fall back to a local Ubuntu GPU desktop environment. |
| Issue | Standard 3DGS geometries bleed across physical boundaries, making strict dimension extraction messy. | Utilize the CNN semantic bounding boxes to force geometric separation between different materials before dimensions are calculated. |
| Dependency | The Final-2 evaluation strictly requires testing with a real-world stakeholder. | Identify and secure a commitment from a local Construction Site Supervisor prior to the Mid-1 evaluation phase. |