FYP Issues

- Check and Verify Synchronization Drift between ARCore Metrics and Video Frames

- The Issue: The Android MediaRecorder takes a fraction of a second to initialize the MP4 encoder, causing the video stream to permanently lag behind the ARCore UI thread logging the IMU data to poses.csv. This results in tracking data being misaligned with the visual frames, causing the unprojection math to smear the geometry into overlapping "slices" across the camera trajectory.

- Mitigation Strategy: Implement a manual POSE_OFFSET variable in the ingestion script to slip the tracking indices forward (e.g., frame_idx + 30) until the video frames and tracking data perfectly align.

- Manage GPU VRAM

- Motion Blur and Sensor Exposure

- The Issue: To track SLAM features indoors, ARCore locks the camera focus. In standard indoor lighting, the smartphone sensor artificially lengthens the shutter exposure time (30 FPS = ~33ms shutter limit) to absorb more light, resulting in severe motion blur during physical panning. Blurred frames destroy both Metric3D depth estimations and 3D Gaussian Splatting texture rendering.

Mitigation Strategy:

- Hardware: Force a 60 FPS recording configuration in the Android app to physically halve the maximum shutter exposure time. Maximize room lighting during capture.

- Software: Utilize a cv2.Laplacian variance filter in the Python ingestion loop to mathematically detect and discard any smeared frames before they reach the GPU.

- Orientation and Coordinate Space Mismatches

  - The Issue: ARCore logs the MP4 and intrinsic data (poses.csv) in the sensor's native landscape format (1920x1080). However, the room scan video is captured vertically, meaning the frames must be rotated for Metric3D to understand the geometry. Furthermore, OpenCV projects into a Y-Down/Z-Forward space, while ARCore operates in an OpenGL Y-Up/Z-Back space.

  - Mitigation Strategy:

    - Rotate the RGB frame 90° Clockwise for Metric3D inference.

    - Rotate the resulting depth map 90° Counter-Clockwise back to landscape to match the poses.csv intrinsics.

    - Invert the Y and Z axes of the calculated coordinates before applying the ARCore rotation matrices.

- Open3D Mmeory Alignment