Final Year Project Proposal

BuildScan 3D

Visual-Inertial 3D Gaussian Splatting for Construction Spatial Verification

Declared Entry Stream: Stream A, Research-Derived

Primary SDG / Thematic Alignment: SDG 9 (Industry, Innovation and Infrastructure)

FAST School of Computing, National University of Computer and Emerging Sciences

Group Members: [To be completed — see Section 12]

Supervisor: [To be confirmed]

Date: August 2026

Table of Contents

(Right-click below and choose “Update Field” — or press F9 — after opening this document in Word, to populate the table of contents.)

# 1. Executive Summary and Project Context

BuildScan 3D is a research-derived (Stream A) final year project that investigates whether a smartphone-based capture pipeline, combining visual and inertial data through 3D Gaussian Splatting (3DGS), can produce web-deliverable, dimensionally verifiable reconstructions of active construction environments. The project studies the engineering required to turn a state-of-the-art reconstruction method into an operable system for construction-site documentation and remote inspection, rather than proposing a new rendering algorithm.

The intended workflow proceeds from mobile video capture on an active construction site, through automated cloud-based reconstruction, to a browser-based interface that allows a remote reviewer to navigate the reconstructed environment and extract spatial measurements. The system targets two stakeholder groups with different operating contexts and requirements, described in detail in Section 2.

| Field | Detail |
| --- | --- |
| Project Title | BuildScan 3D: Visual-Inertial 3D Gaussian Splatting for Construction Spatial Verification |
| Declared Entry Stream | Stream A, Research-Derived. The project begins from published work on 3D Gaussian Splatting and visual-inertial tracking, and studies the engineering required to turn these methods into a deployable system under real capture and delivery constraints. |
| Capture Users (on-site) | Construction site supervisors and quality assurance (QA) inspectors who record video of site conditions. |
| Review Users (remote) | Project managers and civil engineers who inspect the resulting 3D reconstructions and extract dimensions to verify as-built progress. |

Table 1: Project Overview

## SDG / Thematic Alignment

The project aligns with SDG 9 (Industry, Innovation and Infrastructure): by making 3D spatial verification more accessible than dedicated hardware-based reality capture, the system aims to support more reliable documentation of built infrastructure. This alignment provides broader context only; it does not substitute for the Complex Computing Problem, technical contribution, or systematic evaluation established in the sections that follow.

# 2. Problem Formulation and Stakeholder Workflow

## 2.1 Stakeholders

The system is intended to bridge the documentation and verification gap between two stakeholder groups that operate in different environments and with different requirements. The capture user — a construction site supervisor or QA inspector — works in a dynamic, physically constrained on-site environment and needs a fast, low-friction capture tool that does not interrupt routine site work. The review user — a project manager, civil engineer, or investor — works remotely and needs a navigable, spatially accurate model that can be used to verify structural progress against architectural plans without a site visit.

For scope-approval purposes, the supervisor acts as the academic approver of this proposal. Consistent with Section 5.2 of the FAST FYP guide for research-derived (Stream A) projects, the review user described above is treated as the notional client: the system's requirements are defined around what this operator would need to run and trust the system. The on-site construction stakeholder engaged for Semester-2 user acceptance testing (Section 11.3) is a real-world validation contact for this notional client role, and will be formally named once confirmed (Section 12). [Flag: confirm whether this commitment is already secured or still pending before submission — see also Sections 10 and 11.3.]

## 2.2 The Current Workflow

As-built verification and progress monitoring on the sites relevant to this project currently rely on manual 2D photography and basic physical measurement. Site supervisors typically walk active construction floors on a regular basis, capturing large numbers of disconnected 2D photographs to document structural progress, electrical rough-ins, and plumbing routes. Where a sense of scale is required, supervisors commonly hold a tape measure or another reference object within the camera frame before taking a photograph. These photographs are then manually collated into static progress reports and sent to remote managers, with no persistent spatial or dimensional record of the space itself.

## 2.3 The Problem and the Consequence of Failure

This workflow is limited because a 2D photograph does not encode verifiable spatial coordinates. Once infrastructure such as a plumbing chase or an electrical conduit is concealed behind drywall or a concrete pour, a remote reviewer working only from 2D photographs has no way to extract measurements to confirm that the built structure matches the design. Because construction sites change continuously and are sometimes altered without updated documentation, reliance on unverified photographs can also lead reviewers to assume that the physical build matches the design when this has not actually been confirmed.

Rework arising from undetected discrepancies between built and designed structures is widely discussed in construction-management literature as a significant contributor to project cost and schedule overruns, with slower field-to-office information cycles compounding the delay. [A specific, verifiable source for this figure is to be confirmed by the project team before final submission, rather than relying on a general industry estimate.]

## 2.4 The Technical Gap

Three-dimensional scanning can, in principle, address this gap, but the available options each fall short in a way that is relevant to this project. Dedicated LiDAR-based reality-capture hardware produces accurate models but is costly and requires scheduled scan sessions, which limits its use for frequent, informal documentation. Consumer mobile scanning and splatting applications are more accessible, but testing of representative tools indicates two recurring limitations. Photogrammetry-driven mobile scanners can produce geometrically inconsistent reconstructions on plain, low-texture surfaces such as bare drywall, and generally do not provide automated semantic labelling of specific materials. Mobile Gaussian Splatting and NeRF-based content-creation tools produce visually convincing reconstructions but function as closed, black-box outputs that do not support parametric measurement, such as extracting a floor plan or a pipe clearance. Separately, browser-based splat-editing tools (for example, PlayCanvas SuperSplat) demonstrate that cleaning and compressing .ply output and delivering it through a web viewer is technically feasible, which supports the web-delivery approach adopted in this project.

Taken together, these observations point to a gap this project investigates: whether unconstrained, mobile-captured video of a construction environment can be turned into a 3D reconstruction that survives featureless and low-texture surfaces, remains renderable in a standard web browser, and supports dimensional measurement, without requiring dedicated scanning hardware.

# 3. Solution Landscape and Prior-FYP Comparison

To establish the technical gap and support the Complex Computing Problem claim in Section 5, the proposed approach is compared structurally against a dedicated hardware-based reality-capture product, consumer mobile AI scanning applications, the most relevant prior FAST FYP, and the current manual workflow described in Section 2.2. The comparison considers the problem addressed, the target users, the core technical approach, an evidence-based limitation, and the resulting implication for this project.

| Existing Solution / Prior FYP | Problem & Target Users | Core Approach / Strong Features | Evidence-Based Limitation | Implication for This Project |
| --- | --- | --- | --- | --- |
| Commercial reality capture (e.g., Matterport) | Real estate and enterprise facility documentation; standardised, high-accuracy scans. | Dedicated LiDAR hardware producing accurate floor plans and 360° virtual tours. | Requires scheduled scan sessions and dedicated hardware costing several thousand dollars; navigation is limited to fixed viewpoints rather than continuous 3D movement. | The proposed system must avoid proprietary hardware dependency by relying on standard smartphone video to enable rapid, frequent capture. |
| Consumer mobile AI scanners (Polycam, Luma AI) | Content creators and general AEC users; fast visual records and marketing-quality assets. | On-device LiDAR and cloud-based NeRF/3DGS pipelines producing captures in minutes. | Closed, black-box outputs. Reported testing indicates Luma AI does not support measurable floor-plan extraction, and Polycam's photogrammetry can produce geometrically inconsistent walls without semantic material labelling. | The proposed system must integrate 2D-to-3D semantic segmentation to identify materials and automate dimension extraction directly from the reconstructed geometry. |
| Prior FAST FYP (F25-283-D-Roomora) | Homeowners and interior designers; visualising furniture layout before purchase. | Mobile room scanning generating standard 3D meshes combined with AI-based 2D-to-3D conversion and local AR visualisation. | Standard 3D meshes lack the photorealistic fidelity of 3DGS needed to capture complex, cluttered infrastructure, and local AR visualisation does not address compressing and delivering large datasets to a remote web browser. | The proposed system must engineer a cloud-based 3DGS pipeline with ARCore telemetry fusion and .ksplat compression to deliver large reconstructions to remote managers through a browser. |
| Current manual / non-digital workflow | Construction site supervisors; documenting daily as-built progress for remote project managers. | Walking the site taking disconnected 2D photographs, manually holding a tape measure in frame for scale. | 2D photographs lack depth and spatial context, making remote dimensional verification impossible and contributing to costly rework, as discussed in Section 2.3. | The proposed system must automate spatial and dimensional extraction to replace manual, error-prone tape-measure estimation. |

Table 2: Structural Comparison of Existing Solutions and Prior FYP

# 4. Research and Technical Basis

The proposed pipeline builds on published work in neural rendering, visual-inertial tracking, and semantic feature extraction. Table 3 maps the relevant literature to the specific architectural decisions it influences and the evidence the project will produce to validate each decision.

| Source / Method | Main Finding | Relevance & Limitation | Decision Influenced | Planned Evidence |
| --- | --- | --- | --- | --- |
| Kerbl et al., SIGGRAPH / ACM TOG, 2023: 3D Gaussian Splatting for Real-Time Radiance Field Rendering | Introduced 3D Gaussian Splatting, replacing heavy neural-network rendering with anisotropic Gaussians to achieve real-time rendering from sparse point clouds. | Relevance: establishes photorealistic, real-time rendering as feasible from sparse capture. Limitation: raw .ply output is typically hundreds of megabytes and will crash standard mobile web browsers if delivered uncompressed. | Adopt 3DGS as the core reconstruction method rather than standard textured meshes, but require an automated .ksplat compression stage before any web delivery. | File-size reduction benchmark comparing compressed .ksplat output against the raw .ply output for representative captures. |
| VIGS-SLAM, 2025: Visual-Inertial Gaussian Splatting SLAM | Reports that tightly coupling visual correspondences with IMU pre-integration reduces tracking loss in motion blur and low-texture conditions relative to visual-only tracking. | Relevance: directly motivates fusing inertial telemetry with visual tracking for featureless construction surfaces such as bare drywall. Limitation: reported evaluation relies on curated datasets and higher-end stereo hardware than a typical consumer smartphone. | Replace visual-only tracking with fused ARCore telemetry (IMU and spatial anchors) to stabilise camera pose estimation on low-texture surfaces. | Demonstration of camera-pose convergence and reconstruction stability in a low-texture room, compared against a visual-only tracking baseline. |
| ContextGS / Compact 3DGS, 2024–2025 | Reports that heuristic pruning, opacity-gradient densification, and context modelling can substantially reduce the number of 3DGS primitives required for a given scene. | Relevance: directly addresses the GPU-memory and browser-memory bottlenecks that prevent 3DGS delivery on consumer and edge devices. Limitation: reported reductions are scene- and implementation-dependent. | Implement an adaptive Three.js web viewer with level-of-detail and splat-budget management driven by client-side hardware signals. | Sustained frame-rate measurement on a mid-range Android browser under the adaptive viewer, compared against an unoptimised viewer. |
| GSemSplat, 2024: Semantic 3D Gaussian Splatting | Reports a method for linking 2D semantic features and context-aware masks to 3D Gaussian primitives to support open-vocabulary spatial understanding. | Relevance: motivates decoupling semantic labelling from geometric reconstruction rather than training them jointly. Limitation: jointly training semantics and geometry per scene introduces computational overhead not well suited to this project's processing-time constraints. | Run 2D semantic segmentation independently on video keyframes and project the resulting bounding boxes onto the reconstructed 3D geometry after training, rather than jointly optimising semantics and geometry. | Selectable semantic labels (e.g., drywall, wiring) returning measured dimensions in the web viewer, checked against manual measurement on a sample of labelled elements. |

Table 3: Research-to-Decision Mapping

Note: full citation details (authors, venue, DOI) for VIGS-SLAM (2025), ContextGS / Compact 3DGS (2024–2025), and GSemSplat (2024) are to be confirmed against their primary publications before final submission — see the placeholder entries in Section 13, References. The Kerbl et al. (2023) 3D Gaussian Splatting paper is a well-established reference and is cited with high confidence.

# 5. Complex Computing Problem (CCP) Justification

## 5.1 Formal Problem Definition

The core engineering challenge of BuildScan 3D is to formulate a system that extracts navigable, dimensionally usable 3D reconstructions from unconstrained, noisy smartphone video captured on active construction sites. The system must maintain tracking through visual failures on featureless or reflective building surfaces (bare drywall, unfinished concrete) by reconciling drifting inertial telemetry with visual photogrammetry; resolve the competing requirements of high-fidelity volumetric rendering against strict client-side browser memory limits; and derive structured dimensional measurements from unstructured point-cloud geometry, in a setting where spatial estimation errors carry real engineering and financial consequences.

## 5.2 Seoul Accord Characteristics Mapping

In line with the Seoul Accord graduate attributes adopted by the FAST School of Computing, the engineering problem underlying this project exhibits four characteristics that place it beyond routine implementation. As the guide notes, a credible claim need not exhibit every characteristic; the four identified here are those the team can currently evidence, and this list will be revisited if the project's constraints shift during FYP-1.

| Seoul Accord Characteristic | Engineering Conflict / Challenge | Technical Resolution in BuildScan 3D |
| --- | --- | --- |
| 1. Conflicting Requirements | High-fidelity 3D rendering versus strict mobile-browser memory limits. | Automated .ksplat compression combined with an adaptive Three.js splat budget. |
| 3. Depth of Knowledge | 3D coordinate transformations and non-Euclidean point-cloud geometry. | Multi-sensor data fusion and custom 2D-to-3D back-projection. |
| 5. Beyond Standard Practice | Routine web/CRUD architecture cannot handle asynchronous, heavy-compute reconstruction. | GPU task scheduling via a message queue and Dockerised cloud GPU workers. |
| 8. Interdependence | Failure in one pipeline stage cascades through the rest of the system. | End-to-end error tracking and validation checks across pipeline stages. |

Table 4: Seoul Accord Characteristics Mapping

### 5.2.1 Conflicting Requirements

High-fidelity 3D Gaussian Splatting can generate millions of anisotropic Gaussians to represent detailed building elements such as exposed conduits and rough surfaces, producing uncompressed point clouds that can exceed several hundred megabytes. Mobile web browsers, by contrast, operate under strict WebGL memory allocations and limited bandwidth, and will fail to render or will crash when attempting to parse an uncompressed scene of this size. The project resolves this by designing an automated post-processing stage that performs heuristic floater pruning and converts raw .ply output into binary .ksplat streams, paired with an adaptive Three.js rendering engine that adjusts the splat budget according to client-side hardware signals.

### 5.2.2 Depth of Knowledge

Addressing this problem requires working knowledge of multi-view geometry, structure-from-motion mathematics, non-Euclidean point-cloud manipulation, and the dynamics of Gaussian-based radiance fields. In practice, the team must bridge distinct coordinate spaces, integrating six-degree-of-freedom ARCore camera poses (metric scale) with relative COLMAP visual keypoints, and subsequently project 2D semantic pixel masks into 3D Gaussian bounding boxes.

### 5.2.3 Beyond Standard Practice

Standard software engineering practice relies on synchronous web services and conventional CRUD architectures, which are not suited to unconstrained photogrammetric reconstruction in featureless environments. The proposed system instead requires an asynchronous, event-driven orchestration layer (a Redis-backed job queue) that provisions ephemeral cloud GPU instances, executes an iterative 3DGS optimisation process, runs automated point-pruning scripts, and synchronises spatial metadata back to the web portal.

### 5.2.4 Interdependence

The pipeline's stages cannot be treated as isolated modules. The stability of on-device ARCore tracking directly affects the convergence and accuracy of COLMAP camera-pose estimation; pose accuracy in turn affects the geometric fidelity of the 3DGS optimisation; and this fidelity ultimately determines whether the dimensional-extraction stage produces reliable, engineering-grade measurements or distorted spatial approximations. A weakness introduced early in the pipeline therefore propagates through every later stage.

# 6. Proposed Contribution

In one sentence: Existing construction-site 3D capture options either require costly dedicated hardware or lose tracking on the low-texture surfaces typical of active sites, and cannot be delivered through a standard web browser. BuildScan 3D contributes an engineered capture-to-delivery pipeline that fuses inertial and visual tracking, compresses reconstructions for browser delivery, and projects semantic labels onto the geometry to support dimensional measurement — evaluated against these existing systems and against physical tape-measurement baselines.

Existing approaches to construction-site 3D capture either depend on costly dedicated hardware, as with Matterport, or on purely visual mobile pipelines that lose tracking on featureless, low-texture surfaces and cannot be delivered to a standard web browser at usable frame rates, as observed in testing of Polycam and Luma AI (Section 3). BuildScan 3D will contribute an engineered pipeline that (a) fuses ARCore inertial telemetry with COLMAP structure-from-motion to stabilise camera tracking on featureless construction surfaces, (b) applies automated point-pruning and .ksplat compression to keep reconstructions renderable in a standard browser, and (c) performs 2D-to-3D semantic projection to support dimensional measurement directly from the reconstructed geometry. This contribution will be evaluated against the reconstruction behaviour and web-delivery performance of the comparison systems in Section 3, and against physical tape-measurement baselines for dimensional accuracy, as detailed in Section 10.

This is a system and architecture contribution in the sense described by the FAST School of Computing guide: the individual components (3DGS, COLMAP, ARCore, Three.js) are drawn from established research and tooling, but the reconstruction pipeline that fuses them reliably under construction-site constraints, and delivers the result within browser memory limits, is the team's own engineering work.

# 7. Proposed System Overview and Architecture

The proposed system is organised around three sequential stages, corresponding to the two stakeholder groups described in Section 2.1. Table 5 summarises the stage, the responsible actor, the key actions performed, and the resulting output.

| Stage | Actor | Key Actions | Output |
| --- | --- | --- | --- |
| 1. On-site capture and validation | Capture user (site supervisor / QA inspector) | Records a short video segment with the mobile application while the application performs edge-based image-quality checks (motion blur, low light) and prompts the user to correct capture conditions before upload. | A validated video segment with synchronised ARCore telemetry, uploaded to object storage in the background. |
| 2. Cloud processing | Automated backend (no direct user interaction) | A Dockerised GPU worker fuses the ARCore telemetry with the video via COLMAP, trains the 3D Gaussian point cloud, runs 2D semantic segmentation on keyframes, projects labels onto the geometry, and compresses the result. | A compressed, semantically labelled .ksplat scene stored for retrieval by the web dashboard. |
| 3. Remote review and measurement | Review user (project manager / civil engineer) | Opens the web dashboard, navigates the reconstructed environment in the browser viewer, and selects labelled elements to extract physical dimensions. | On-screen dimensional measurements and a navigable record of as-built site conditions. |

Table 5: System Workflow Overview

Workflow walkthrough. The following traces the three stages of Table 5 in concrete terms, following a single capture from the site to the reviewer’s screen.

On-site capture. The supervisor opens the Android application in an unfinished room and begins recording. Before and throughout the recording, the application continuously analyses the live camera feed and surfaces an immediate on-screen warning — for example, prompting the user to enable the flashlight or slow down — if the room is too dark or the phone is moved too quickly, so that a poor take is caught before it consumes upload bandwidth or cloud compute. Once a clean, roughly 60-second segment has been captured, it uploads to object storage in the background while the supervisor continues with other site work.

Cloud processing. On receiving the upload, the backend automatically provisions a Dockerised GPU worker on Ubuntu Linux. The worker fuses the ARCore pose data with the video via COLMAP, trains the 3D Gaussian point cloud, runs the 2D CNN over keyframes to detect materials such as drywall and wiring, and prunes and compresses the result into a web-optimised .ksplat file — all without further input from either stakeholder.

Remote review. The project manager opens the Next.js web dashboard, navigates the reconstructed room in three dimensions, and uses the “X-Ray / Rough-In” toggle to reveal wiring or plumbing since concealed behind finished walls. Selecting a labelled element returns its physical dimensions directly, without a site visit.

## 7.1 System Architecture

At a component level, the system comprises four cooperating layers: a mobile capture application (Android, using ARCore for pose and IMU telemetry); a backend orchestration layer (Node.js/Express, coordinating a Redis-backed job queue); a Dockerised GPU reconstruction worker (running COLMAP for structure-from-motion, the 3D Gaussian Splatting training and compression pipeline, and 2D semantic segmentation); and a web-based review layer (a Next.js backend and a Three.js/WebGL viewer). A lightweight metadata store, tracking scans, users, and semantic annotations, sits alongside the compressed .ksplat scene storage so that the web layer can query scan history without re-reading full reconstruction files.

Figure 1: BuildScan 3D component and data-flow architecture, across the three workflow stages of Table 5.

This diagram is a working draft reflecting the components already named in the functional requirements of Section 8; it is presented here to establish that the workflow can be sketched end to end at proposal stage, as required by Section 6 of the FAST FYP guide. It will be refined into a complete component and data-flow diagram, with defined interfaces and failure-handling behaviour, as part of the software design specification at Mid-1.

# 8. System Requirements

## 8.1 Functional Requirements

Requirements are grouped by the three workflow stages introduced in Section 7.

### Workflow A: Edge Capture and Validation

- FR1 (ARCore Telemetry Extraction): The Android application shall record 1080p video while simultaneously extracting six-degree-of-freedom ARCore spatial anchors and IMU telemetry into a synchronised JSON file.

- FR2 (Real-Time Capture Validation): The mobile client shall implement edge-based Image Quality Assessment (IQA) using Laplacian variance (motion blur) and lux estimation (exposure), and shall present real-time on-screen guidance — for example, prompting the user to enable the flashlight or slow down — to correct movement or lighting before upload.

- FR3 (Asynchronous Upload): The mobile application shall upload the raw video and telemetry data to object storage in the background without blocking the user interface.

### Workflow B: Distributed Orchestration and Reconstruction

- FR4 (Distributed Job Queuing): The backend shall enqueue uploaded scan data into a Redis-backed message broker to asynchronously provision and orchestrate Dockerised GPU cloud workers.

- FR5 (Absolute Spatial Anchoring): The cloud pipeline shall fuse ARCore tracking data with video frames via COLMAP to align the generated 3D Gaussian point cloud to a consistent physical coordinate frame.

- FR6 (3DGS Reconstruction): The GPU worker shall execute the 3D Gaussian Splatting training process to transform the video and ARCore poses into a raw 3D point-cloud reconstruction.

- FR7 (2D-to-3D Semantic Projection): A 2D convolutional neural network (CNN) shall detect specified construction materials — such as drywall and electrical wiring — in video keyframes, and the resulting bounding boxes shall be projected onto the 3D Gaussians to label the geometry.

- FR8 (Heuristic Pruning and Compression): The post-processing stage shall remove low-opacity artefact splats and compress the .ply output into a web-optimised .ksplat binary.

### Workflow C: Remote Delivery and Measurement

- FR9 (Progressive Web Rendering): The web dashboard shall stream .ksplat files into a Three.js WebGL viewer supporting continuous, first-person 3D navigation.

- FR10 (Semantic Dimension Extraction): The web viewer shall calculate and display physical surface-area and length measurements for a user-selected, semantically labelled element.

- FR11 (Temporal “X-Ray” Overlay, planned, contingent on FR7): Where an earlier, rough-in-stage scan of the same space exists, the web viewer shall provide an “X-Ray / Rough-In” toggle that overlays the prior scan's semantic regions — for example, wiring or plumbing later concealed behind finished walls — onto the current scan, using the shared coordinate frame established by FR5. This capability is planned for Semester 2 and its scope will be finalised once the reliability of the semantic labelling pipeline (FR7) has been evaluated.

## 8.2 Non-Functional Requirements

The following non-functional targets represent the thresholds the team will design toward. They will be measured empirically during Mid-2 and Final-2 testing; where evidence gathered during FYP-1 indicates a target is not achievable within the available hardware and time budget, the deviation will be recorded and justified through the change-control procedure described in the FAST FYP guide.

| NFR | Target |
| --- | --- |
| NFR1: Capture Efficiency | The mobile application shall maintain a stable video capture rate of at least 30 FPS while processing background telemetry. |
| NFR2: Edge Processing Latency | Real-time motion-blur and lighting detection shall execute on the Android client in under 16 ms per frame, to avoid dropped frames. |
| NFR3: Pipeline Latency | End-to-end cloud processing of a 60-second video shall complete within 15 minutes on an RTX 4090-class cloud GPU instance. |
| NFR4: Compression Ratio | .ksplat binary compression shall reduce the raw 3D file payload by at least 60% relative to the raw .ply output. |
| NFR5: Client Accessibility | The web viewer shall maintain at least 30 FPS on standard mid-range browsers via an adaptive splat budget. |
| NFR6: Dimensional Accuracy | Physical measurements extracted from the 3D geometry shall be within 5% error relative to physical tape-measurement baselines. |

Table 6: Non-Functional Requirements

# 9. Proposal-Stage Proof of Concept (POC) Evidence

The highest-risk assumption for this project is whether a 3DGS reconstruction, once pruned and compressed into the .ksplat format, can be rendered smoothly inside a standard web browser rather than causing a memory failure. The team's plan for testing this assumption is to record a short video of a low-texture indoor space, process it into a raw 3D Gaussian point cloud on a local GPU or a cloud notebook environment, compress the output into a .ksplat file, and confirm that it renders in a browser-based viewer, recording the resulting file-size reduction as evidence.

[SECTION TO BE COMPLETED: Proposal-Stage Proof of Concept (POC) Evidence. Insert here: (1) the risk assumption tested; (2) the input and setup used (e.g., source video, hardware, software); (3) the result obtained, including the raw .ply file size and the compressed .ksplat file size; (4) what the result confirmed or disproved; (5) what it changed in the design, scope, or plan; and (6) supporting evidence such as a screenshot of the reconstruction rendering in a browser. This section is mandatory for the proposal defense per Section 12 of the FAST FYP Idea Selection and Proposal Readiness Guide, and a proposal without demonstrated feasibility evidence should not normally be approved.]

# 10. Evaluation Plan

Evaluation is planned around three related but distinct questions, consistent with Section 13.1 of the FAST FYP guide: whether the implemented system behaves as specified (testing), whether it meaningfully addresses the problem relative to a baseline (evaluation), and whether the intended users accept it for real use (acceptance).

- Baseline / comparator: the current manual, tape-measure-based workflow described in Section 2.2, together with the qualitative comparison against Matterport, Polycam, and Luma AI established in Table 2.

- Data and scenarios: representative site captures spanning varied lighting conditions and surface types (including featureless drywall and reflective or cluttered surfaces), to test tracking stability rather than only favourable conditions.

- Metrics: the non-functional targets in Section 8.2 (capture rate, processing latency, compression ratio, viewer frame rate, dimensional accuracy), together with qualitative comparison of reconstruction fidelity against Table 2.

- Failure cases: tracking loss on featureless or reflective surfaces, browser failure on an unpruned or under-compressed scene, and incorrect material classification affecting downstream dimension extraction.

- User acceptance testing: a supervised task walkthrough conducted in Semester 2 with the on-site construction stakeholder secured by the project team, following the required documentation of method, participants, tasks and results, issues found, and the recorded acceptance outcome set out in Section 13.1 of the guide. [Flag: see the confirmation note in Section 11.3.]

The conclusions drawn from this evaluation will be stated narrowly and limited to what the evidence supports; a successful end-to-end demonstration alone will not be treated as sufficient evidence of dimensional accuracy or reliability.

# 11. Scope, Risks, and Dependencies

## 11.1 Committed Scope Across Two Semesters

To provide a verifiable grading standard, the scope is divided across the two semesters as follows.

### Semester 1 (FYP-1): Core Build and Cloud Pipeline

- Mobile capture: Android application with ARCore telemetry extraction and the edge-based image-quality assessment module for lighting and blur detection (FR1–FR3).

- Cloud orchestration: backend service using a Redis-backed queue for asynchronous job handling (FR4).

- Infrastructure: Dockerised cloud environments, provisioned on Ubuntu Linux, for parallel GPU compute.

- Reconstruction engine: fusing ARCore telemetry with COLMAP and generating a raw 3D Gaussian point cloud (FR5, FR6).

- Exit gate (Final-1): a working core pipeline that takes a mobile video, validates its quality, and generates a spatially scaled 3D scene.

### Semester 2 (FYP-2): Optimisation, Intelligence, and Delivery

- Compression engineering: heuristic pruning scripts converting raw .ply files into .ksplat binaries (FR8).

- Semantic AI integration: 2D-to-3D projection mapping material labels onto the 3D geometry, and the planned “X-Ray / Rough-In” overlay (FR7, FR10, FR11).

- Web dashboard: the Next.js and MongoDB-backed review portal featuring the adaptive Three.js viewer (FR9).

- Evaluation: formal user acceptance testing with the named construction stakeholder, per Section 10.

- Exit gate (Final-2): a fully integrated, deployed web platform with verifiable dimension extraction, having undergone UAT.

### Optional Extension (not committed to the core grade)

- Automated report generation comparing extracted 3D scan dimensions against uploaded 2D floor plans.

## 11.2 External Component and Dependency Classification

Following Section 14 of the FAST FYP guide, external components used by the project are classified as follows.

- Support (disclosure only, no further justification expected): Next.js and Node.js/Express (backend framework and runtime), MongoDB (database), Ubuntu Linux and Docker (containerisation and orchestration tooling), Redis (job queue), and the Cloudflare R2 SDK (object storage). These provide standard scaffolding and do not supply the claimed contribution.

- Core-assist (permitted, with justification and validation): COLMAP (base structure-from-motion), the 3DGS CUDA training kernels, standard pretrained YOLO/Mask2Former CNN weights for material detection, and the Three.js rendering library. The team is responsible for integrating, adapting, and orchestrating these components; the contribution (the ARCore/COLMAP fusion, the .ksplat compression pipeline, and the semantic projection logic) remains identifiable independently of them.

- Core-replacement: none identified. The ARCore multi-sensor fusion, edge-based image-quality assessment, asynchronous orchestration, .ksplat compression, and 2D-to-3D semantic mapping are engineered by the team rather than supplied by an external component.

## 11.3 Risk, Assumption, Issue, and Dependency (RAID) Register

| Type | Description | Mitigation Strategy |
| --- | --- | --- |
| Risk | A user captures unusable, blurred, or dark video, wasting GPU cloud compute time. | Edge-based image-quality assessment (FR2) blocks poor uploads and prompts real-time capture correction before recording ends. |
| Risk | ARCore hardware telemetry drifts over time, causing spatial warping in the reconstructed model. | Mobile capture is limited to short (approximately 60-second) segments to limit accumulated IMU drift and to allow periodic spatial resets. |
| Assumption | The cloud GPU provider (RunPod) will remain highly available for processing. | The pipeline is containerised (Docker), allowing execution to fall back to a local GPU environment if the cloud provider is unavailable. |
| Issue | Standard 3DGS geometry can bleed across physical boundaries, complicating precise dimension extraction. | Semantic bounding boxes from FR7 are used to enforce geometric separation between distinct materials before dimensions are calculated. |
| Dependency | Final-2 evaluation requires testing with a real construction stakeholder. | A local construction-site stakeholder has been engaged by the team. [Flag: verify this is accurate at time of submission — the reviewed source material describes this commitment as not yet secured. Reconcile before finalising.] Formal confirmation of their role and the UAT scheduling details will be recorded in Section 12 once the team roster and supervisor are finalised. |

Table 7: Risk, Assumption, Issue, and Dependency (RAID) Register

# 12. Team Responsibilities and Individual Technical Ownership

This section will record, per member, the primary technical ownership area, the shared integration and quality responsibilities, and the named on-site stakeholder confirmed for Semester 2 evaluation, consistent with Section 5 of the FAST FYP Idea Selection and Proposal Readiness Guide.

[SECTION TO BE COMPLETED: Team Responsibilities and Individual Technical Ownership. For each group member, record: name and registration number; primary technical ownership area (for example, mobile capture and edge validation; cloud orchestration and the reconstruction pipeline; semantic extraction and web delivery); the shared integration, quality-assurance, and repository responsibilities held jointly by the team; and confirmation that the division of work passes the "drop test" in Section 5 of the guide: removing any one member would remove a meaningful, evidenced technical responsibility. Also record here the name and role of the confirmed on-site construction stakeholder referenced in Sections 10 and 11.3, and the basis on which they agreed to participate in Semester 2 user acceptance testing.]

# 13. References

- Kerbl, B., Kopanas, G., Leimkühler, T., & Drettakis, G. (2023). 3D Gaussian Splatting for Real-Time Radiance Field Rendering. ACM Transactions on Graphics, 42(4).

- VIGS-SLAM (2025). Visual-Inertial Gaussian Splatting SLAM. [Full citation to be confirmed by the project team before final submission.]

- ContextGS / Compact 3DGS (2024–2025). [Full citation to be confirmed by the project team before final submission.]

- GSemSplat (2024). Semantic 3D Gaussian Splatting. [Full citation to be confirmed by the project team before final submission.]

- FAST School of Computing, National University of Computer and Emerging Sciences. FYP Idea Selection and Proposal Readiness Guide.

- Seoul Accord, Section D: Graduate Attributes.